MATLAB SIMULATION OF SUPERHETERODYNE RECEIVER  FOR AM COMMUNICATION SYSTEM WITH FREQUENCY  MIXING AND FILTERING

Objectives

i) To model and simulate the operation of a superheterodyne receiver for an AM communication  system using MATLAB by implementing frequency mixing and intermediate frequency (IF)  signal generation.

ii) To recover the original message signal through intermediate frequency filtering and envelope  detection, and analyze the waveforms of the message, carrier, AM, IF, and recovered signals.

PROGRAM (MATLAB CODE)

clc;

clear;

close all;

% Parameters

fs = 100000;

t = 0:1/fs:0.02;

fm = 500; % Message frequency

fc = 10000; % Carrier frequency

fLO = 14550; % Local Oscillator (IF = 4550 Hz)

% Message Signal

m = cos(2*pi*fm*t);

% Carrier Signal

c = cos(2*pi*fc*t);

% AM Signal

am = (1 + m).*cos(2*pi*fc*t);

% Local Oscillator

LO = cos(2*pi*fLO*t);

% Frequency Mixing

mixed = am .* LO;

% IF Filter

[b,a] = butter(4,5000/(fs/2));

IF = filter(b,a,mixed);

% Envelope Detection

env = abs(hilbert(IF));

% Display

figure;

subplot(4,1,1)

plot(t,m)

title('Message Signal')

grid on

figure;

subplot(5,1,2)

plot(t,c)

title('Carrier Signal')

grid on

subplot(5,1,3)

plot(t,am)

title('AM Signal')

grid on

subplot(5,1,4)

plot(t,IF)

title('Intermediate Frequency (IF) Signal') grid on

subplot(5,1,5)

plot(t,env)

title('Recovered Signal')

grid on

OUTPUT:

Result

The superheterodyne receiver was successfully simulated in MATLAB, and the message signal,  carrier signal, AM signal, Intermediate Frequency (IF) signal, and recovered signal were  obtained through frequency mixing and filtering.