POWER SPECTRAL DENSITY (PSD) ANALYSIS OF PHASE  MODULATED (PM) SIGNALS USING MATLAB

Objectives

i) To generate a Phase Modulated (PM) signal in MATLAB using specified carrier and message  signal parameters.

ii) To compute the frequency spectrum of the PM signal using the Fast Fourier Transform (FFT). iii) To calculate and plot the Power Spectral Density (PSD) of the PM signal using MATLAB. iv) To identify the carrier component and sideband frequencies present in the PM spectrum. v) To analyze the effect of phase modulation on the spectral distribution and bandwidth of the  transmitted signal.

vi)To investigate the influence of modulation index on the Power Spectral Density characteristics  of the PM signal.

PROGRAM (MATLAB CODE)

clc;

clear;

close all;

% Parameters

Am = 1; % Message Amplitude (V)

fm = 100; % Message Frequency (Hz)

Ac = 5; % Carrier Amplitude (V)

fc = 1000; % Carrier Frequency (Hz)

fs = 20*fc; % Sampling Frequency (Hz)

kp = pi/2; % Phase Sensitivity (rad/V)

% Time Vector

t = 0:1/fs:4/fm;

% Message Signal

m = Am*sin(2*pi*fm*t);

% PM Signal

pm = Ac*cos(2*pi*fc*t + kp*m);

% Phase Deviation

beta = kp*Am;

% FFT

N = length(pm);

f = linspace(-fs/2,fs/2,N);

PM_FFT = abs(fftshift(fft(pm)))/N;

% Power Spectral Density

PSD = (abs(fftshift(fft(pm))).^2)/(N*fs);

% Approximate Bandwidth

BW = 2*(beta+1)*fm;

fprintf('Phase Deviation = %.2f radians\n',beta);

fprintf('Estimated Bandwidth = %.2f Hz\n',BW);

% Plotting

figure('Name','PSD Analysis of PM Signal','NumberTitle','off'); subplot(2,2,1);

plot(t,m);

title('Message Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(2,2,2);

plot(t,pm);

title('Phase Modulated Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(2,2,3);

plot(f,PM_FFT);

title('Frequency Spectrum of PM Signal');

xlabel('Frequency (Hz)');

ylabel('Magnitude');

grid on;

subplot(2,2,4);

plot(f,PSD);

title('Power Spectral Density');

xlabel('Frequency (Hz)');

ylabel('PSD');

grid on;

OUTPUT:

RESULT

The Power Spectral Density (PSD) of the Phase Modulated (PM) signal was  successfully obtained and analyzed using MATLAB.

i) Message Frequency = 100 Hz

ii) Carrier Frequency = 1000 Hz

iii) Phase Deviation = 1.57 radians

iv) PM Modulation Index = 1.57

v) Estimated Bandwidth = 514 Hz

vi) The PSD plot showed the carrier component and multiple sidebands around  the carrier frequency, confirming the frequency-domain characteristics of Phase  Modulation.