MATLAB-BASED MODELLING OF VSB MODULATOR AND  DEMODULATOR FOR ANALOG TELEVISION COMMUNICATION APPLICATIONS

Objectives:

i) To generate and analyse the Vestigial Sideband (VSB) modulated signal by performing  DSB-SC modulation and applying a suitable filter using MATLAB.

ii) To implement coherent demodulation and recover the original message signal from the VSB  signal, and compare the transmitted and recovered waveforms.

PROGRAM (MATLAB CODE)

clc;

clear;

close all;

% Parameters

fs = 100000; % Sampling Frequency t = 0:1/fs:0.02; % Time Vector

fm = 500; % Message Frequency fc = 5000; % Carrier Frequency

% Message Signal

m = cos(2*pi*fm*t);

% Carrier Signal

c = cos(2*pi*fc*t);

% DSB-SC Modulation

dsb = m .* c;

% VSB Filter (Low-pass + High-pass Approximation) [b,a] = butter(4, fc/(fs/2));

vsb = filter(b,a,dsb);

% Coherent Demodulation

demod = vsb .* (2*cos(2*pi*fc*t));

% Recover Message

[b1,a1] = butter(4, 1000/(fs/2));

rec = filter(b1,a1,demod);

% Display Results

figure;

subplot(5,1,1)

plot(t,m)

title('Message Signal')

grid on

subplot(5,1,2)

plot(t,c)

title('Carrier Signal')

grid on

subplot(5,1,3)

plot(t,dsb)

title('DSB-SC Signal')

grid on

subplot(5,1,4)

plot(t,vsb)

title('VSB Signal')

grid on

subplot(5,1,5)

plot(t,rec,'r')

title('Recovered Message')

grid on

output:

Result 1:

i) The VSB modulated signal was successfully generated from the DSB-SC signal using  MATLAB and verified through the displayed waveforms.

ii) The original message signal was successfully recovered through coherent demodulation and  low-pass filtering, confirming the correct implementation of the VSB communication system.