Characteristics Study of Side Band Power and  Frequencies in AM Using MATLAB

Objectives:

i) To generate an Amplitude Modulated (AM) signal in MATLAB using specified carrier and  modulating signal parameters.

ii) To determine and verify the upper sideband (USB) and lower sideband (LSB) frequencies of  the AM signal using frequency spectrum analysis in MATLAB.

iii) To calculate the carrier power, sideband power, and total transmitted power of the AM signal  for a given modulation index using MATLAB.

Objective 1:

To generate an Amplitude Modulated (AM) signal in MATLAB using specified carrier and  modulating signal parameters

Code:

clc;

clear;

close all;

Am=1;

fm=500;

Ac=5;

fc=5000;

fs=20*fc;

t=0:1/fs:4/fm;

m = Am*sin(2*pi*fm*t);

c = Ac*sin(2*pi*fc*t);

mu = Am/Ac;

am = Ac*(1+mu*sin(2*pi*fm*t)).*sin(2*pi*fc*t);

dem = abs(hilbert(am)); dem = dem-mean(dem);

N=length(am); f=linspace(-fs/2,fs/2,N);

AM_FFT=abs(fftshift(fft(am)))/N;

USB=fc+fm; LSB=fc-fm;

Pc=Ac^2/2; Psb=(mu^2*Pc)/4;

figure('Name','AM Modulation','NumberTitle','off');

subplot(3,2,1), plot(t,m), title('Message'), grid on

subplot(3,2,2), plot(t,c), title('Carrier'), grid on

subplot(3,2,3), plot(t,am), title('AM Signal'), grid on

subplot(3,2,4), plot(t,dem), title('Demodulated'), grid on

subplot(3,2,5), plot(f,AM_FFT), xlim([fc-3*fm fc+3*fm]), title('Sidebands'), grid on subplot(3,2,6), bar([Pc Psb Psb]), set(gca,'XTickLabel',{'Carrier','USB','LSB'}), title('Power'), grid on

fprintf('μ=%.2f | USB=%d Hz | LSB=%d Hz | Pc=%.2f W | Psb=%.3f  W\n',mu,USB,LSB,Pc,Psb);

Output: