CALCULATION AND VERIFICATION OF FREQUENCY  DEVIATION AND MODULATION INDEX OF FM SIGNALS USING  MATLAB SIMULATION

Objectives:

1. To generate a Frequency Modulated (FM) signal in MATLAB using specified carrier and  message signal parameters.

2. To calculate and verify the frequency deviation of the FM signal from the given carrier  frequency and modulating signal parameters.

3. To determine and verify the modulation index of the FM signal using the relationship  between frequency deviation and message frequency.

4. To analyze the effect of message frequency on frequency deviation and modulation  index, and verify their relationship through MATLAB simulation

PROGRAM (MATLAB CODE)

% Calculation and Verification of Frequency Deviation

% and Modulation Index of FM Signal

clc;

clear;

close all;

%% Message Signal Parameters

Am = 2; % Message amplitude (V)

fm = 500; % Message frequency (Hz)

%% Carrier Signal Parameters

Ac = 1; % Carrier amplitude (V)

fc = 10000; % Carrier frequency (Hz)

%% FM Parameters

kf = 250; % Frequency sensitivity (Hz/V)

fs = 100000; % Sampling frequency (Hz)

T = 0.01; % Simulation duration (s) %% Time Vector

t = 0:1/fs:T;

%% Message Signal

m = Am*cos(2*pi*fm*t);

%% Frequency Deviation

delta_f = kf*Am;

%% Modulation Index

beta = delta_f/fm;

%% FM Signal

fm_signal = Ac*cos(2*pi*fc*t + beta*sin(2*pi*fm*t)); %% Display Parameters

fprintf('Carrier Frequency (fc) = %.0f Hz\n',fc); fprintf('Message Frequency (fm) = %.0f Hz\n',fm); fprintf('Message Amplitude (Am) = %.0f V\n',Am); fprintf('Frequency Sensitivity (kf) = %.0f Hz/V\n',kf);

fprintf('\nFrequency Deviation (Delta f) = %.2f Hz\n',delta_f); fprintf('Modulation Index (Beta) = %.2f\n',beta); %% Verification

beta_verify = delta_f/fm;

fprintf('Verified Modulation Index = %.2f\n',beta_verify); %% Verification Check

tolerance = 1e-6;

if abs(beta - beta_verify) < tolerance

fprintf('\nVerification Successful\n');

else

fprintf('\nVerification Failed\n');

end

%% Plotting

figure;

subplot(2,1,1);

plot(t,m,'LineWidth',1.5);

grid on;

title('Message Signal');

xlabel('Time (s)');

ylabel('Amplitude (V)');

subplot(2,1,2);

plot(t,fm_signal,'LineWidth',1);

grid on;

title('FM Signal');

xlabel('Time (s)');

ylabel('Amplitude (V)');

OUTPUT:

Result

1. Generate and visualize carrier, message, and FM signals using MATLAB.

2. Calculate the theoretical frequency deviation and verify it from the simulated FM waveform.

3. Calculate the FM modulation index and verify the relationship β = Δf/fm.

4. Interpret the effect of changing message frequency on modulation index and verify that, for constant  frequency sensitivity and message amplitude, the peak frequency deviation remains constant.