PERFORMANCE ANALYSIS OF FM SYSTEM UNDER GAUSSIAN  WHITE NOISE USING MATLAB

Objectives

1. To generate a Frequency Modulated (FM) signal in MATLAB using the specified carrier and  message signal parameters.

2. To introduce Additive White Gaussian Noise (AWGN) into the generated FM signal and simulate  the effect of a noisy communication channel using MATLAB.

3. To analyze the effect of AWGN on the transmitted FM signal by comparing the original and noisy  FM signals using time-domain and frequency-domain analysis.

PROGRAM (MATLAB CODE)

clc;

clear;

close all;

% Parameters

fs = 100000;

t = 0:1/fs:0.02;

fm = 500; % Message frequency

fc = 5000; % Carrier frequency

kf = 100; % Frequency sensitivity

% Message Signal

m = cos(2*pi*fm*t);

c = cos(2*pi*fc*t);

% FM Modulation

fm_sig = cos(2*pi*fc*t + 2*pi*kf*cumsum(m)/fs);

% Add Gaussian White Noise

noisy = awgn(fm_sig,10,'measured'); % 10 dB SNR

% Display

figure;

subplot(4,1,1)

plot(t,m)

title('Message Signal')

grid on

subplot(4,1,2)

plot(t,m)

title('Carrier Signal')

grid on

subplot(4,1,3)

plot(t,fm_sig)

title('FM Signal')

grid on

subplot(4,1,4)

plot(t,noisy)

title('FM Signal with Gaussian White Noise')

grid on

OUTPUT:

RESULT

1. Generated and visualized an FM signal using specified carrier and message signal parameters. 2. Introduced AWGN into the FM signal at a specified signal-to-noise ratio (SNR). 3. Compared clean and noisy FM signals in the time domain.

4. Computed the FFT and compare the frequency-domain characteristics of the clean and noisy FM signals.

5. Evaluated the influence of AWGN on the quality and spectral characteristics of the transmitted FM  signal.

LABORATOR